PUT9000

#W#W#W#W#W#W#W

What is Put9000?

#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W

Put9000 is a MSX-DOS2 TSR that will enable you to use fast full colour ANSI

on the Gfx9000!

What do I need to use Put9000?

#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W

The requirements of Put9000 are :

- At least MSX2 or better

- A Gfx9000 from Sunrise Swiss or a V9990 evaluation board from Yamaha

- MSX-DOS2

- At least one free segment for the TSR

How does this program do the trick?

#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W#W

It does some dirty, non standard, trick. It changes some data in one of the

system segments of MSX-DOS2. It redirects a routine in one of the system

pages to call a subroutine in another segment. This routine normally call the

BIOS to put a character on screen. Instead it will run through the TSR.

This TSR checks if the screen mode is 255. Is it 255, then it will put

character on the Gfx9000 screen. If not, the normal procedure will be

followed to allow normal programs to work properly under non-Gfx9000 modes.

PUT9000.COM will load PUT9000.OVL into a free segment, sets this segment to

system segment, redirects a routine in the DOS2 system segments, initialises

the driver and finaly sets the screen to 255.

Put9000 is able to detect wheter or not Put9000 is already resident, if so,

it will find out in what page Put9000 resides, and then overinstalls it

properly. So it can be used more than once without problems.

With normal use, it happens that a program sets the screen mode. When this

happens, Put9000 will be deactivated.

To manually (de)ativate Put9000, there are two programs on this disk.

VON.COM  sets the screen mode to 255, thus enabling Put9000

VOFF.COM sets the screen mode to 0, thus disabling Put9000

This program is written by : Erik Maas

For contact :

- Email 	: erik.maas@tip.nl

- Snail mail	: Erik Maas

Woudseweg 5a

5275 JH Den Dungen

- BBS		: The Games BBS

tel. +31-412-640358

Leave a message to : ERIK MAAS

###############################################################################